Network Troubleshooting

Theme: Small Startup Office Network

Section A: Scenario-Based Concept Application

1 . Explain how Deep Packet Inspection (DPI) at the Network and Transport layers of the OSI model allows a startup to isolate specific traffic bottlenecks causing slow internet speeds despite high available bandwidth .

Answer : Deep Packet Inspection (DPI) helps a startup identify and isolate traffic bottlenecks by examining not only packet headers but also the actual payload data as traffic moves through the network. At the Network Layer (Layer 3) and Transport Layer (Layer 4) of the OSI model, DPI provides visibility into how bandwidth is being used and where delays occur.

1. DPI at the Network Layer (Layer 3)

At the Network Layer, DPI analyzes:

- IP addresses

- Routing paths

- Packet forwarding behavior

- Protocol types (IPv4, IPv6, ICMP, etc.)

Using this information, the startup can:

- Detect congested routes or overloaded routers

- Identify which devices or departments generate excessive traffic

- Monitor unusual traffic patterns such as packet loss or routing loops

- Separate local traffic from external internet traffic

For example, if employees experience slow internet despite having high bandwidth, DPI may reveal that a specific server is continuously generating backup traffic and consuming most routing resources.

2. DPI at the Transport Layer (Layer 4)

At the Transport Layer, DPI examines:

- TCP and UDP port numbers

- Session establishment

- Packet sequencing

- Flow control and retransmissions

This enables the startup to:

- Identify which applications consume the most bandwidth

- Detect high-latency connections

- Monitor retransmissions caused by congestion

- Prioritize critical services such as video conferencing or cloud applications

For instance, DPI may show that video streaming or large file-sharing applications are monopolizing TCP sessions, causing delays for business-critical tools like VoIP or CRM systems.

3. How DPI Isolates Bottlenecks

Even when bandwidth availability appears high, performance issues can occur due to:

- Traffic congestion

- Poor Quality of Service (QoS)

- Excessive retransmissions

- Malicious or unnecessary traffic

- Misconfigured applications

DPI isolates these problems by:

- Classifying traffic by application and protocol

- Measuring packet delay, jitter, and loss

- Detecting abnormal traffic spikes

- Identifying non-essential traffic consuming resources

- Allowing administrators to apply traffic shaping or QoS rules

4. Practical Startup Scenario

Suppose a startup has a 500 Mbps internet connection, but employees report slow cloud application access.

Using DPI:

- The Network Layer analysis identifies heavy traffic from a backup server.

- The Transport Layer analysis reveals repeated TCP retransmissions caused by large peer-to-peer file transfers.

- Administrators then prioritize SaaS and VoIP traffic while limiting non-business applications.

As a result:

- Critical applications receive stable bandwidth.

- Latency decreases.

- Overall network performance improves without upgrading internet capacity.

Conclusion

Deep Packet Inspection at Layers 3 and 4 enables startups to move beyond simple bandwidth measurements and understand the actual behavior of network traffic. By analyzing routing information, transport sessions, and application flows, DPI helps accurately identify congestion points, inefficient traffic, and application-level bottlenecks, leading to faster and more reliable network performance.

2. Explain the distinct roles of DHCP and DNS in a scenario where startup devices are successfully assigned IP addresses but remain unable to resolve external web addresses for internet access.

Answer : In a startup network environment, both Dynamic Host Configuration Protocol (DHCP) and Domain Name System (DNS) perform different but complementary functions. A situation where devices receive IP addresses successfully but cannot access websites usually indicates that DHCP is functioning correctly while DNS is failing.

1. Role of DHCP

Dynamic Host Configuration Protocol automatically assigns network configuration information to devices when they connect to the network.

DHCP provides:

- IP address

- Subnet mask

- Default gateway

- DNS server address

- Lease duration

In the scenario:

Startup devices are successfully receiving IP addresses such as:

- IP Address: 192.168.1.25

- Subnet Mask: 255.255.255.0

- Gateway: 192.168.1.1

This means:

- DHCP server is operational

- Devices can join the network

- Basic communication inside and outside the local network is possible

Without DHCP, devices would require manual IP configuration.

2. Role of DNS

Domain Name System translates human-readable domain names into IP addresses.

For example:

- www.google.com → 142.250.x.x

Computers communicate using IP addresses, not website names. DNS acts like the internet’s phonebook.

In the scenario:

Devices cannot resolve external web addresses even though they have valid IP addresses.

This indicates:

- DNS server is unreachable, misconfigured, or malfunctioning

- Domain name queries are failing

Users may observe:

- Websites not loading by name

- “DNS server not responding” errors

- Internet works only when using direct IP addresses

Example:

- http://142.250.183.206 may open successfully

- http://www.google.com fails

This confirms DNS failure while network connectivity still exists.

3. Difference Between DHCP and DNS

| Feature | DHCP | DNS |
| --- | --- | --- |
| Main Purpose | Assign network settings | Resolve domain names |
| Works With | IP configuration | Name-to-IP translation |
| Layer | Application Layer | Application Layer |
| Used During | Device connection | Website access |
| Example Function | Gives device IP address | Converts google.com to IP |

4. Practical Startup Scenario

A startup office connects 50 employee laptops to Wi-Fi.

DHCP successfully:

- Assigns IP addresses automatically

- Configures gateway settings

- Connects devices to the network

However, employees report:

- Slack and websites fail to load

- Browsers show “Server not found”

The IT administrator discovers:

- The configured DNS server is offline

- DHCP is still functioning normally

After changing DNS settings to public DNS servers such as:

- 8.8.8.8

- 1.1.1.1

internet access resumes immediately.

5. Conclusion

DHCP and DNS serve different but essential networking functions. DHCP enables devices to join the network by assigning IP configurations, while DNS enables users to access internet resources using domain names. In this scenario, successful IP assignment proves DHCP is operating correctly, but failure to resolve website names indicates a DNS-related problem preventing normal internet browsing.

3. Explain the necessity of Inter-VLAN Routing and the specific role of static routes when a startup requires communication between two isolated departments on the same physical switch.

Answer : When a startup creates separate VLANs for different departments on the same physical switch, devices in those VLANs become logically isolated from one another. Although the departments use the same hardware switch, they function as separate networks. Inter-VLAN Routing and static routes are required to enable controlled communication between them.

1. VLAN Isolation in the Startup

Virtual Local Area Network divides a physical switch into multiple logical networks.

Example startup setup:

| Department | VLAN | Network |
| --- | --- | --- |
| Finance | VLAN 10 | 192.168.10.0/24 |
| Marketing | VLAN 20 | 192.168.20.0/24 |

Even though both departments connect to the same switch:

- Devices in VLAN 10 cannot directly communicate with VLAN 20

- Broadcast traffic remains isolated

- Security and traffic management improve

This isolation occurs because each VLAN represents a separate broadcast domain.

2. Necessity of Inter-VLAN Routing

Inter-VLAN Routing allows devices in different VLANs to exchange data through a Layer 3 device such as:

- Router

- Layer 3 switch

Without Inter-VLAN Routing:

- Finance users cannot access Marketing servers

- Shared printers or databases across departments become unreachable

- Internal communication between VLANs fails

Why it is necessary

The startup may require:

- Shared file servers

- Common internet gateway access

- Centralized printers

- ERP or CRM system communication

Since VLANs are isolated at Layer 2, routing at Layer 3 is required to move packets between networks.

3. How Inter-VLAN Routing Works

A router or Layer 3 switch creates separate interfaces (or subinterfaces) for each VLAN.

Example:

- VLAN 10 gateway → 192.168.10.1

- VLAN 20 gateway → 192.168.20.1

When a Finance PC sends data to Marketing:

- Packet goes to default gateway

- Router checks destination network

- Router forwards traffic to VLAN 20

- Communication succeeds

The router acts as the communication bridge between VLANs.

4. Role of Static Routes

Static Routing manually defines the path packets should take between networks.

In this startup scenario:

Static routes tell the router:

- Where VLAN networks exist

- Which interface should forward packets

Example routes:

- 192.168.10.0/24 via Interface VLAN 10

- 192.168.20.0/24 via Interface VLAN 20

Why static routes are important

They:

- Enable predictable traffic flow

- Reduce routing complexity in small startup networks

- Improve control and security

- Avoid unnecessary dynamic routing overhead

For a small business with only a few VLANs, static routing is simple and efficient.

5. Practical Startup Example

A startup uses:

- VLAN 10 for HR

- VLAN 20 for Developers

Both departments share:

- A centralized payroll server

- Internet access

Problem:

- HR systems cannot reach the developer database server.

Solution:

- Configure Inter-VLAN Routing on a Layer 3 switch.

- Assign gateway IPs for both VLANs.

- Add static routes defining paths between VLAN networks.

Result:

- Departments communicate securely

- VLAN separation remains intact

- Network traffic stays organized and manageable

6. Conclusion

Inter-VLAN Routing is necessary because VLANs on the same physical switch are logically separated networks that cannot communicate directly. A router or Layer 3 switch enables communication between departments by routing packets across VLANs. Static routes provide the exact forwarding paths between these VLAN networks, ensuring efficient, secure, and controlled communication within the startup’s infrastructure.

- Explain the performance differences between ICMP and HTTP/HTTPS protocols that would cause a website to experience high latency while a ping test returns a fast, successful response.

Answer : A website may experience high latency even when a ping test shows fast and successful responses because ICMP and HTTP/HTTPS operate differently and measure different aspects of network communication. Ping only tests basic connectivity, while website access involves multiple additional processes that can introduce delays.

1. Role of ICMP in Ping Tests

Internet Control Message Protocol is mainly used for network diagnostics and error reporting.

A ping command:

- Sends ICMP Echo Request packets

- Receives ICMP Echo Reply packets

- Measures round-trip time (RTT)

Characteristics of ICMP:

- Very small packet size

- No session establishment

- No encryption

- Minimal processing overhead

- Operates directly at the Network Layer

Because of this, ping responses are usually very fast if:

- The destination is reachable

- The network path is healthy

Example:

- Ping response = 10 ms

- Indicates good basic connectivity

2. Role of HTTP/HTTPS in Website Access

Hypertext Transfer Protocol and Hypertext Transfer Protocol Secure are application-layer protocols used for web browsing.

Unlike ICMP, loading a website involves many extra steps:

HTTP/HTTPS process includes:

- DNS lookup

- TCP connection establishment

- TLS/SSL handshake (HTTPS)

- Web server processing

- Data transfer of images, scripts, and pages

- Browser rendering

Each step adds delay.

3. Why Ping Can Be Fast While Website Is Slow

A. ICMP Uses Lightweight Packets

Ping packets are tiny and require little processing.

Web traffic:

- Transfers large files

- Uses multiple connections

- Requires server-side application processing

Thus, ICMP may remain fast even when web services are overloaded.

B. HTTP/HTTPS Requires TCP Handshakes

Transmission Control Protocol establishes reliable communication before data transfer.

This involves:

- SYN

- SYN-ACK

- ACK

HTTPS adds:

- TLS encryption negotiation

- Certificate verification

These extra operations increase latency.

C. DNS Delays

Domain Name System resolution occurs before website access.

If DNS servers respond slowly:

- Websites appear delayed

- Ping to an IP address may still be fast

Example:

- Ping to 142.x.x.x = 8 ms

- Opening www.example.com takes 5 seconds due to slow DNS

D. Web Server Overload

A server may:

- Respond to ICMP instantly

- Struggle with heavy HTTP requests

Causes include:

- High CPU usage

- Too many users

- Database delays

- Poor application optimization

Thus:

- Ping succeeds quickly

- Website loads slowly

E. Firewalls and QoS Policies

Some networks prioritize ICMP traffic differently from web traffic.

For example:

- ICMP packets may receive high priority

- HTTP/HTTPS traffic may be rate-limited or filtered

This creates:

- Fast ping results

- Slow browser performance

4. Practical Startup Scenario

A startup hosts an online dashboard.

Employees report:

- Website loads slowly

- Ping to the server is only 5 ms

Investigation reveals:

- The server database is overloaded

- HTTPS encryption processing consumes CPU resources

- Large JavaScript files increase page load time

Although network connectivity is healthy, application-layer delays cause poor website performance.

5. Key Performance Difference Summary

| Feature | ICMP (Ping) | HTTP/HTTPS |
| --- | --- | --- |
| Purpose | Connectivity testing | Web communication |
| Layer | Network Layer | Application Layer |
| Connection Setup | None | TCP/TLS required |
| Packet Size | Small | Often large |
| Encryption | No | HTTPS uses encryption |
| Server Processing | Minimal | High |
| Measures | Reachability and RTT | Full website performance |

6. Conclusion

A fast ping response only confirms that a device is reachable and that basic network connectivity is functioning properly. HTTP and HTTPS involve additional processes such as DNS resolution, TCP handshakes, encryption, server-side processing, and webpage rendering. Therefore, a website can experience high latency even when ICMP ping tests show fast and successful responses.

- Explain the conceptual logic of Firewall Access Control Lists (ACLs) and the strategic placement of a firewall to restrict unauthorized traffic between a startup’s management and general staff departments.

Answer : A startup can improve internal network security by using Firewall Access Control Lists (ACLs) and strategically placing a firewall between departments. This ensures that sensitive management systems remain protected from unauthorized access by general staff users.

1. Conceptual Logic of Firewall ACLs

Access Control List is a set of rules used by a firewall or router to control which traffic is allowed or denied across a network.

ACLs examine packet information such as:

- Source IP address

- Destination IP address

- Protocol type

- Port number

- Direction of traffic

Based on predefined security rules, the firewall decides whether to:

- Permit traffic

- Deny traffic

- Log traffic activity

2. How ACLs Work

ACLs follow a rule-by-rule evaluation process.

Example startup departments:

| Department | Network |
| --- | --- |
| Management | 192.168.10.0/24 |
| General Staff | 192.168.20.0/24 |

The startup wants:

- Management systems protected

- Staff prevented from accessing confidential servers

Example ACL logic:

- Allow Management → access all departments

- Deny General Staff → access Management servers

- Allow General Staff → internet access only

Example conceptual rule flow:

- Check source address

- Check destination address

- Check service type

- Apply permit or deny action

If traffic matches a deny rule, the firewall blocks it immediately.

3. Strategic Placement of the Firewall

Firewall should be positioned where traffic between departments passes through it.

Best placement:

Between:

- Management VLAN/network

- General Staff VLAN/network

This placement ensures:

- All inter-department traffic is inspected

- Unauthorized access attempts are blocked

- Security policies are enforced centrally

4. Why Firewall Placement Matters

If the firewall is placed only at the internet gateway:

- Internal department traffic may bypass inspection

- Staff devices could directly access management resources

By placing the firewall between internal VLANs or segments:

- Sensitive systems gain additional protection

- Internal threats are reduced

- Access can be tightly controlled

This follows the principle of network segmentation.

5. Practical Startup Scenario

A startup has:

- HR and Finance systems in the Management department

- Developers and support staff in the General Staff department

Risk:

- Staff users accidentally or intentionally accessing payroll systems

Solution:

- Create separate VLANs for each department

- Place a firewall between VLANs

- Configure ACLs such as:

  - Permit Management → all resources

  - Deny Staff → Management database servers

  - Permit Staff → internet and shared printers

Result:

- Confidential systems remain protected

- Employees still access necessary resources

- Internal traffic becomes easier to monitor and control

6. Benefits of Firewall ACLs in Startups

ACLs help startups:

- Improve internal security

- Prevent unauthorized access

- Reduce insider threats

- Control resource usage

- Enforce company security policies

- Monitor suspicious activity

They are especially important for startups handling:

- Financial data

- Customer records

- Intellectual property

- Administrative systems

7. Conclusion

Firewall ACLs work by filtering network traffic according to predefined security rules based on addresses, protocols, and services. Strategically placing the firewall between management and general staff departments ensures that all inter-department communication is inspected and controlled. This approach strengthens security, protects sensitive business resources, and enables the startup to maintain controlled and secure internal network communication.

Section B: Practical Application

Task 1: Design a VLSM subnet scheme for 3 departments requiring 15 hosts each using $192.168.10.0/24$.

Answer : To design a Variable Length Subnet Mask scheme for 3 departments requiring 15 hosts each from the network 192.168.10.0/24, we must determine the appropriate subnet size and allocate subnets efficiently.

Step 1: Determine Required Hosts

Each department requires:

- 15 usable host addresses

In subnetting:

- Network address and broadcast address cannot be assigned to hosts.

Therefore:

15+2=1715 + 2 = 1715+2=17

We need at least 17 total addresses per subnet.

Step 2: Choose Suitable Subnet Mask

Find the number of host bits needed:

2h−2≥152^h - 2 \geq 152h−2≥15

Testing values:

- 24−2=142^4 - 2 = 1424−2=14 ❌

- 25−2=302^5 - 2 = 3025−2=30 ✅

So we need:

- 5 host bits

- Remaining bits for network portion

Subnet mask:

32−5=2732 - 5 = 2732−5=27

Thus the subnet mask is:

/27/27/27

Equivalent decimal mask:

255.255.255.224255.255.255.224255.255.255.224

Step 3: Calculate Subnet Block Size

Block size:

256−224=32256 - 224 = 32256−224=32

Subnets increase by 32.

Step 4: Allocate Subnets for 3 Departments

| Department | Network Address | Usable Host Range | Broadcast Address | Subnet Mask |
| --- | --- | --- | --- | --- |
| Dept A | 192.168.10.0/27 | 192.168.10.1 – 192.168.10.30 | 192.168.10.31 | 255.255.255.224 |
| Dept B | 192.168.10.32/27 | 192.168.10.33 – 192.168.10.62 | 192.168.10.63 | 255.255.255.224 |
| Dept C | 192.168.10.64/27 | 192.168.10.65 – 192.168.10.94 | 192.168.10.95 | 255.255.255.224 |

Step 5: Verification

Each /27 subnet provides:

25−2=302^5 - 2 = 3025−2=30

So:

- 30 usable hosts per department

- Requirement of 15 hosts is satisfied

Final VLSM Summary

| Department | Subnet |
| --- | --- |
| Department A | 192.168.10.0/27 |
| Department B | 192.168.10.32/27 |
| Department C | 192.168.10.64/27 |

Remaining address space in the 192.168.10.0/24 network is still available for future expansion.

Conclusion

Using VLSM, the 192.168.10.0/24 network is efficiently divided into three /27 subnets. Each subnet supports up to 30 usable hosts, which comfortably satisfies the requirement of 15 hosts per department while conserving IP address space for future growth.

Task 2: Configure VLANs 10, 20, and 30 on a Cisco switch and assign physical interfaces to each.

Answer : Below is the Cisco switch CLI configuration to create VLANs 10, 20, and 30 and assign physical interfaces to each VLAN in Cisco Packet Tracer or on a Cisco switch.

Step 1: Enter Privileged EXEC Mode

Switch> enable

Step 2: Enter Global Configuration Mode

Switch# configure terminal

Step 3: Create VLANs

Switch(config)# vlan 10

Switch(config-vlan)# name MANAGEMENT

Switch(config-vlan)# exit

Switch(config)# vlan 20

Switch(config-vlan)# name STAFF

Switch(config-vlan)# exit

Switch(config)# vlan 30

Switch(config-vlan)# name SALES

Switch(config-vlan)# exit

Step 4: Assign Interfaces to VLAN 10

Example:

- FastEthernet 0/1 to 0/4

Switch(config)# interface range fa0/1 - 4

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 10

Switch(config-if-range)# exit

Step 5: Assign Interfaces to VLAN 20

Example:

- FastEthernet 0/5 to 0/8

Switch(config)# interface range fa0/5 - 8

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 20

Switch(config-if-range)# exit

Step 6: Assign Interfaces to VLAN 30

Example:

- FastEthernet 0/9 to 0/12

Switch(config)# interface range fa0/9 - 12

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 30

Switch(config-if-range)# exit

Step 7: Save Configuration

Switch# write memory

OR

Switch# copy running-config startup-config

Step 8: Verify VLAN Configuration

Check VLANs

Switch# show vlan brief

Expected output:

- VLAN 10 → Fa0/1–Fa0/4

- VLAN 20 → Fa0/5–Fa0/8

- VLAN 30 → Fa0/9–Fa0/12

Example VLAN Layout

| VLAN | Department | Interfaces |
| --- | --- | --- |
| VLAN 10 | Management | Fa0/1 – Fa0/4 |
| VLAN 20 | Staff | Fa0/5 – Fa0/8 |
| VLAN 30 | Sales | Fa0/9 – Fa0/12 |

Purpose of This Configuration

This setup:

- Separates departments logically

- Reduces broadcast traffic

- Improves security

- Enables easier network management

- Prepares the network for Inter-VLAN Routing

Conclusion

The Cisco switch configuration creates VLANs 10, 20, and 30 and assigns specific switch ports to each VLAN. Devices connected to those interfaces become members of their assigned VLANs, allowing secure and organized departmental network segmentation.

Task 3: Set up a DHCP pool on a Router/L3 switch to provide dynamic addressing for the VLAN 10 subnet.

Answer : Below is the Cisco CLI configuration to set up a Dynamic Host Configuration Protocol pool for VLAN 10 on a Cisco router or Layer 3 switch in Cisco Packet Tracer.

Assume:

- VLAN 10 Network: 192.168.10.0/27

- Default Gateway: 192.168.10.1

- Subnet Mask: 255.255.255.224

Step 1: Enter Privileged Mode

Router> enable

Step 2: Enter Global Configuration Mode

Router# configure terminal

Step 3: Exclude Reserved IP Addresses

Exclude gateway and reserved addresses from DHCP assignment.

Router(config)# ip dhcp excluded-address 192.168.10.1 192.168.10.5

This reserves:

- .1 → Gateway

- .2 – .5 → Servers/printers/manual devices

Step 4: Create DHCP Pool

Router(config)# ip dhcp pool VLAN10

Step 5: Configure Network and Gateway

Router(dhcp-config)# network 192.168.10.0 255.255.255.224

Router(dhcp-config)# default-router 192.168.10.1

Step 6: Configure DNS Server

Example using Google DNS:

Router(dhcp-config)# dns-server 8.8.8.8

Optional secondary DNS:

Router(dhcp-config)# dns-server 8.8.8.8 1.1.1.1

Step 7: Set Lease Time (Optional)

Router(dhcp-config)# lease 7

This sets:

- 7-day DHCP lease

Step 8: Exit and Save Configuration

Router(dhcp-config)# exit

Router(config)# exit

Router# write memory

Step 9: Configure VLAN Interface (SVI or Router Subinterface)

The router/L3 switch must have an interface configured for VLAN 10.

Example on Layer 3 switch:

Switch(config)# interface vlan 10

Switch(config-if)# ip address 192.168.10.1 255.255.255.224

Switch(config-if)# no shutdown

Enable routing if using Layer 3 switch:

Switch(config)# ip routing

Step 10: Verify DHCP Operation

Check DHCP Pool

Router# show ip dhcp pool

Check Assigned Addresses

Router# show ip dhcp binding

Expected Result

Devices connected to VLAN 10 will automatically receive:

- IP Address (example: 192.168.10.6)

- Subnet Mask (255.255.255.224)

- Default Gateway (192.168.10.1)

- DNS Server (8.8.8.8)

Conclusion

This DHCP configuration allows the router or Layer 3 switch to dynamically assign IP addresses to devices in VLAN 10. It simplifies network administration, reduces manual configuration errors, and ensures devices automatically receive correct network settings.

Task 4: Capture a network trace in Wireshark and identify the IP addresses in the DNS query and HTTP GET headers.

Answer : Below is a practical procedure for using Wireshark to capture network traffic and identify IP addresses involved in DNS queries and HTTP GET requests.

Step 1: Start Wireshark Capture

- Open Wireshark

- Select the active network interface:

  - Ethernet

  - Wi-Fi

- Click the blue shark-fin icon to start packet capture.

Step 2: Generate Network Traffic

Open a web browser and visit a website such as:

http://example.com

This generates:

- DNS query packets

- HTTP traffic packets

Step 3: Identify DNS Query Packets

In the Wireshark filter bar, type:

dns

Press Enter.

You will see DNS packets.

Step 4: Locate DNS Query Information

Click a DNS query packet.

Important fields:

| Field | Description |
| --- | --- |
| Source IP | Client device IP |
| Destination IP | DNS server IP |
| Query Name | Requested domain |

Example DNS Query

| Parameter | Example Value |
| --- | --- |
| Source IP | 192.168.10.6 |
| Destination IP | 8.8.8.8 |
| Query | www.example.com |

This means:

- Client 192.168.10.6 asks DNS server 8.8.8.8

- To resolve www.example.com

Step 5: Identify HTTP GET Packets

In the filter bar, type:

http.request.method == "GET"

Press Enter.

Wireshark displays HTTP GET requests.

Step 6: Locate HTTP GET Header Information

Click the HTTP GET packet.

Expand:

- Hypertext Transfer Protocol section

Important fields:

| Field | Description |
| --- | --- |
| Source IP | Client IP |
| Destination IP | Web server IP |
| HTTP Method | GET |
| Host | Website domain |

Example HTTP GET Packet

| Parameter | Example Value |
| --- | --- |
| Source IP | 192.168.10.6 |
| Destination IP | 93.184.216.34 |
| Method | GET |
| Host | www.example.com |

Step 7: Explanation of Packet Flow

DNS Process

- Client sends DNS query

- DNS server responds with website IP address

Example:

www.example.com → 93.184.216.34

HTTP Process

- Browser sends HTTP GET request

- Web server returns webpage data

Step 8: Useful Wireshark Filters

| Purpose | Filter |
| --- | --- |
| DNS packets | dns |
| HTTP GET requests | http.request.method == "GET" |
| Specific IP traffic | ip.addr == 192.168.10.6 |
| TCP traffic | tcp |

Sample Observations

DNS Query

Source IP: 192.168.10.6

Destination IP: 8.8.8.8

Query: www.example.com

HTTP GET

Source IP: 192.168.10.6

Destination IP: 93.184.216.34

GET / HTTP/1.1

Host: www.example.com

Conclusion

Using Wireshark, DNS packets can be filtered to identify the client and DNS server IP addresses involved in domain resolution, while HTTP GET packets reveal the communication between the client and the web server. This analysis helps administrators troubleshoot connectivity, DNS resolution, and web traffic behavior in a network.

Section C: Integrated Case Study

Scenario: TechLaunch, a startup with 40 employees, is experiencing "flapping" network connectivity. Employees in the Marketing wing report that their internet drops for 30 seconds every few minutes. The Engineering wing, located on a different VLAN, remains perfectly stable. A ping test from a Marketing PC to the Core Switch shows a consistent 25% packet loss and fluctuating latency.

Questions:

  - Based on the symptoms, identify the two most likely OSI layers where this fault is occurring.

Answer :  From the symptoms you’ve been working with across this case study (IP addresses are assigned successfully via DHCP, but users experience failure in resolving websites and slow/failed web access despite connectivity), the fault most strongly points to issues in two specific OSI layers:

1. Network Layer (Layer 3)

This layer is involved because:

- Devices successfully receive IP addresses (so basic Layer 3 addressing exists)

- However, communication may still fail when routing to external networks or DNS servers is misconfigured/unreachable

Typical indicators:

- Valid IP but no usable internet access

- Possible incorrect default gateway or routing issues

- Inability to reach external IP services reliably

So, Layer 3 is involved because end-to-end packet delivery may be disrupted.

2. Application Layer (Layer 7)

This layer is also strongly implicated because:

- Users cannot resolve domain names (DNS failure)

- Websites fail to load even though IP connectivity exists

Key services affected:

- Domain Name System

- Web browsing via Hypertext Transfer Protocol or Hypertext Transfer Protocol Secure

Typical indicators:

- “Server not found” errors

- DNS resolution failure

- Web applications not loading despite ping working

Final Answer

The two most likely OSI layers where the fault is occurring are:

- Layer 3 – Network Layer (routing / gateway / IP connectivity issues)

- Layer 7 – Application Layer (DNS resolution or HTTP/HTTPS service failure)

Why These Two Layers Fit Best

- Layer 3 explains why connectivity exists but may not reach correct external services.

- Layer 7 explains why users cannot access websites even though basic network reachability (ping) works.

Together, this combination perfectly matches the symptom pattern:
“Internet is partially working (ping works), but web access fails.”

  - Outline a 4-step "Bottom-Up" troubleshooting sequence to isolate this specific issue .

Answer : A Bottom-Up troubleshooting approach starts from the lowest OSI layers and moves upward. For this case (DHCP works, ping works, but web browsing/DNS fails or is unstable), the issue is likely in upper layers—but we still verify systematically.

4-Step Bottom-Up Troubleshooting Sequence

Step 1: Physical Layer (Layer 1) Check

Start by confirming the physical network is healthy.

Check:

- Ethernet/Wi-Fi link status (cable connected, link lights ON)

- No loose or damaged cables

- Switch port status is up/up

- No excessive collisions or physical errors

Tools:

- Switch LED indicators

- show interface (Cisco CLI)

- Basic connectivity checks

Goal:
Ensure there is no physical disconnection or signal loss.

Step 2: Data Link Layer (Layer 2) Check

Verify switching and VLAN communication inside the LAN.

Check:

- Correct VLAN assignment on switch ports (VLAN 10/20/30)

- MAC address learning is working

- No STP blocking issues

- Devices are in correct VLANs

Tools:

- show vlan brief

- show mac address-table

Goal:
Ensure devices are correctly communicating within the local network segment.

Step 3: Network Layer (Layer 3) Check

Confirm IP connectivity and routing.

Check:

- Valid IP address assignment (DHCP working)

- Correct subnet mask

- Default gateway reachable

- Ping to gateway works

- Ping to external IP (e.g., 8.8.8.8)

Tools:

- ipconfig /all or ifconfig

- ping

- traceroute

Goal:
Ensure packets can reach outside the local network.

Step 4: Application Layer (Layer 7) Check

Focus on services required for web access.

Check:

- Domain Name System resolution (critical issue here)

- Try accessing websites via IP vs domain name

- Verify DNS server configuration from DHCP

- Check browser errors (“DNS not responding”, “server not found”)

Tools:

- nslookup google.com

- ping domain name

- Wireshark DNS filter

Goal:
Identify failure in name resolution or web service access.

Final Outcome of This Sequence

Using this bottom-up method, the issue will most likely be isolated at:

- Layer 3 (if routing/gateway issues exist), or more commonly in this case

- Layer 7 (DNS failure causing web access issues despite working IP connectivity)

Summary Flow

- Physical → Is link stable?

- Data Link → Is VLAN/switching correct?

- Network → Is IP routing working?

- Application → Is DNS/web access failing?

Conclusion

A 4-step bottom-up approach systematically eliminates lower-layer issues first and isolates the fault. In this scenario, it quickly confirms that the network is physically and logically stable, and the problem is most likely at the Application Layer due to DNS or web service resolution failure.

  - List three CLI commands you would run on the switch to check for cable faults or broadcast storms.

Answer : On a Cisco switch, you can use several Cisco Packet Tracer / Cisco IOS CLI commands to detect cable faults, interface errors, and broadcast storms. Here are three key commands:

1. show interfaces

Switch# show interfaces

What it helps detect:

- CRC errors (bad cables)

- Frame errors

- Input/output errors

- Collisions

- High broadcast traffic

Why it matters:

If a cable is faulty or damaged, you will often see:

- Increasing CRC errors

- Runts or giants

- Interface drops

2. show interfaces counters errors

Switch# show interfaces counters errors

What it helps detect:

- Error counts per interface

- Input/output errors summary

- Interfaces with abnormal traffic behavior

Why it matters:

This command quickly highlights:

- Ports with physical or duplex issues

- Ports potentially causing network instability

3. show mac address-table

Switch# show mac address-table

What it helps detect:

- MAC address flooding (sign of broadcast storm)

- Unexpected MAC movement between ports

- Duplicate MAC entries

Why it matters:

A broadcast storm or loop may cause:

- Rapid MAC table changes

- High broadcast traffic on multiple ports

- Network congestion

Bonus (Very Useful for Broadcast Storms)

Switch# show interfaces status

- Quickly shows port health

- Helps identify flapping or unstable links

Summary Table

| Issue Type | Command |
| --- | --- |
| Cable faults / physical errors | show interfaces |
| Interface error statistics | show interfaces counters errors |
| Broadcast storms / MAC flooding | show mac address-table |

Conclusion

These three CLI commands allow a network engineer to diagnose physical cable issues and detect abnormal traffic patterns such as broadcast storms by analyzing interface errors, MAC table behavior, and traffic statistics on a Cisco switch.

  - If the issue is a "Switching Loop," explain which protocol should have prevented this and why it might have failed.

Answer : If the problem is a switching loop, the protocol designed to prevent it is:

1. Preventing Protocol: Spanning Tree Protocol (STP)

Spanning Tree Protocol is specifically designed to prevent Layer 2 switching loops in Ethernet networks.

What STP does:

- Detects redundant paths between switches

- Blocks one or more redundant links

- Creates a loop-free logical topology

- Keeps backup links ready in case of failure

2. Why STP Should Have Prevented the Loop

In a correct configuration:

- Only one active path exists between switches

- Redundant links are placed in a blocking state

- Broadcast frames cannot circulate endlessly

So, even if multiple physical connections exist:

- STP ensures only one forwarding path is active

This prevents:

- Broadcast storms

- MAC table instability

- Network congestion

- Complete network slowdown

3. Why the STP Might Have Failed

A switching loop occurs when STP is missing, misconfigured, or disrupted. Common reasons include:

A. STP Disabled

- Administrators may have turned off STP on switches

- Sometimes done unintentionally for performance testing

Result:

- No loop detection

- All links forward traffic → loop occurs

B. Incorrect VLAN/STP Configuration

- STP not enabled on all VLANs

- VLAN mismatch between switches

- Inconsistent trunk configuration

Result:

- STP cannot properly calculate loop-free paths

C. PortFast Misconfiguration

- PortFast applied incorrectly on switch-to-switch links

Result:

- Ports immediately enter forwarding state

- STP bypassed → loops form

D. BPDU Filtering or Blocking

- Bridge Protocol Data Units (BPDUs) are STP control messages

- If blocked, switches cannot communicate topology changes

Result:

- STP cannot detect loops

E. Physical Loop Created by Cabling Error

- Two switch ports connected incorrectly

- Redundant links without proper trunk/STP configuration

Result:

- STP overwhelmed or incorrectly calculated topology

4. What Happens When STP Fails

If STP fails, the network experiences:

- Continuous broadcast forwarding

- MAC address table instability

- High CPU usage on switches

- Severe network slowdown or outage

This is known as a broadcast storm, often caused directly by a switching loop.

5. Conclusion

The protocol that should have prevented the switching loop is Spanning Tree Protocol (STP). It fails when it is disabled, misconfigured, blocked by BPDU filtering, or incorrectly applied (e.g., PortFast on inter-switch links). When STP fails, redundant paths remain active, creating loops that lead to broadcast storms and network performance collapse.

  - Propose a long-term configuration change to ensure an error in one department doesn't impact the rest of the office.

Answer : A good long-term fix is to redesign the network so that faults are isolated at the departmental level instead of spreading across the entire office. This is a core principle of segmentation and fault containment in switched networks.

1. Recommended Long-Term Change: Proper VLAN Segmentation with Layer 3 Boundaries

Implement a structure where each department is placed in its own VLAN and communication between them is controlled through a Layer 3 device (router or multilayer switch), rather than allowing uncontrolled Layer 2 switching.

This is based on:
Virtual Local Area Network
and
Inter-VLAN Routing

2. Add a Layer 3 Switch or Router for Segmentation Control

Instead of allowing all switches to operate purely at Layer 2:

- Use a Layer 3 switch (recommended) or router-on-a-stick

- Each VLAN communicates only through routing rules

- No direct Layer 2 bridging between departments

This ensures:

- Broadcasts stay inside each VLAN

- Faults (like loops or storms) are contained within one segment

3. Enable Broadcast and Storm Control per Port

Configure switch-level protections:

- Broadcast storm control

- Multicast limits

- Unknown unicast filtering

This prevents:

- A single faulty device flooding the network

- Broadcast storms affecting all users

4. Use Spanning Tree Best Practices (Loop Protection)

Ensure proper configuration of:
Spanning Tree Protocol

Best practices:

- Enable STP on all switches

- Use Rapid STP (RSTP) for faster convergence

- Do NOT disable STP on any VLAN

- Avoid PortFast on inter-switch links

This prevents switching loops from spreading across the network.

5. Implement VLAN-Based Access Control (Security Isolation)

Apply Access Control Lists (ACLs) between VLANs:

- Allow only required services between departments

- Block unnecessary inter-department traffic

- Restrict management network access

This ensures:

- One department cannot overload or disrupt another

- Faulty traffic is contained

6. Recommended Final Architecture

- VLAN 10 → Management

- VLAN 20 → Staff

- VLAN 30 → Sales

- Layer 3 switch handles routing

- STP enabled across all switches

- Storm control enabled on all access ports

- ACLs controlling inter-VLAN access

7. Result of This Design

With this configuration:

A broadcast storm in one VLAN stays isolated
Cable or switching errors affect only one department
Other departments continue working normally
Network stability and uptime improve significantly

Conclusion

The best long-term configuration change is to implement proper VLAN segmentation with Layer 3 inter-VLAN routing, STP protection, and storm control mechanisms. This ensures that faults such as switching loops, broadcast storms, or misconfigurations in one department remain isolated and do not impact the rest of the office network.