Assignment Module : -7: Network fundamental –

Multiple Choice Question

1- Which of the following messages in the DHCP process are broadcasted? (Choose two)

A. Request

B. Offer

C. Discover

D. Acknowledge

ANSWER : A ) Request  And  C ) Discover

2- Which command would you use to ensure that an ACL does not block web-based TCP traffic?

A. permit any

B. permit tcp any any eq 80

C. permit tcp any eq 80

D. permit any any eq tcp

ANSWER : B ) permit tcp any any eq 80

Question and Answer

3) Explain Network Topologies

Answer : Network topology refers to the physical or logical arrangement of devices (nodes) and connections in a computer network. It defines how computers, servers, switches, and cables are laid out and how data flows between them. A network topology is the physical or logical layout of a network.

Below are the main types of network topologies, with explanations, advantages.

Common types:

- Bus Topology: All devices share one backbone cable. Simple but failure affects entire network.

Advantages:

- Easy to install

- Low cost

- Requires less cable

- Star Topology: Devices connect to a central switch/hub. Easy to manage; most common.

Advantages:

- Easy to add or remove devices

- Failure of one cable does not affect others

- Easy to manage and troubleshoot

- Ring Topology: Devices form a closed loop. Data travels in one direction.

Advantages:

- Equal access for all devices

- Performs well under heavy load

- Mesh Topology : Every device connects to multiple devices. Very reliable but expensive.

Advantages:

- Very reliable and fault tolerant

- High security and redundancy

- Tree Topology : A combination of star and bus topologies, arranged in a hierarchical structure.

Advantages:

- Scalable

- Easy to manage large networks

- Hybrid Topology: Combination of two or more topologies.

Advantages:

- Flexible and scalable

- Reliable

4) Explain TCP/IP Networking Model

ANSWER : The TCP/IP model is a 4-layer framework used for internet communication.

The TCP/IP Networking Model is a conceptual framework used to design and implement computer networks, especially the Internet. It defines how data is transmitted, routed, and received across networks.

The TCP/IP model consists of 4 layers, each with specific functions and protocols.

- Application Layer: User applications (HTTP, FTP, SMTP, DNS)

Purpose:
Provides network services directly to user applications.

Functions:

- Data formatting

- Application-level communication

- User interface to the network

Common Protocols:

- HTTP / HTTPS (web browsing)

- FTP (file transfer)

- SMTP, POP3, IMAP (email)

- DNS (domain name resolution)

- Transport Layer: End-to-end communication (TCP, UDP)

Purpose:
Ensures reliable or fast delivery of data between devices.

Functions:

- End-to-end communication

- Flow control

- Error control

- Data segmentation and reassembly

Main Protocols:

- TCP (Transmission Control Protocol):

  - Reliable, connection-oriented

  - Ensures data delivery, order, and error correction

- UDP (User Datagram Protocol):

  - Fast, connectionless

  - No guarantee of delivery or order

- Internet Layer: Logical addressing and routing (IP, ICMP)

Purpose:
Handles logical addressing and routing of data packets across networks.

Functions:

- Packet routing

- IP addressing

- Path determination

Common Protocols:

- IP (Internet Protocol – IPv4, IPv6)

- ICMP (error reporting)

- ARP (address resolution)

- Network Access Layer: Physical transmission (Ethernet, ARP)

Purpose:
Controls hardware-level transmission of data.

Functions:

- Physical addressing (MAC address)

- Framing

- Error detection

- Data transmission over the physical medium

Technologies/Protocols:

- Ethernet

- Wi-Fi

- ARPANET

- PPP

Advantages of TCP/IP Model

- Highly scalable

- Interoperable across different systems

- Foundation of the Internet

- Protocol-independent

Comparison with OSI Model

- TCP/IP has 4 layers, OSI has 7 layers

- TCP/IP is practical and widely implemented

- OSI is more theoretical

Conclusion

The TCP/IP Networking Model standardizes how data is transmitted over networks and forms the backbone of modern internet communication.

5) Explain LAN and WAN Network

ANSWER : LAN (Local Area Network) : A LAN is a network that connects computers and devices within a small geographical area, such as a home, office, school, or campus.

Characteristics:

- Covers a limited area (few meters to a few kilometers)

- High data transfer speed

- Usually privately owned and managed

- Low latency and high reliability.

Common Technologies:

- Ethernet

- Wi-Fi

Advantages:

- Fast communication

- Easy to set up and manage

- Low cost

- High security

Disadvantages:

- Limited geographical coverage

- Requires maintenance

Examples:

- Home Wi-Fi network

- Office network

- School computer lab

WAN (Wide Area Network) : A WAN is a network that connects devices over a large geographical area, such as cities, countries, or continents.

Characteristics:

- Covers a very large area

- Slower than LAN (generally)

- Uses public or leased communication lines

- Higher latency and cost

Common Technologies:

- Leased lines

- Satellite links

- MPLS

- Internet

Advantages:

- Enables long-distance communication

- Connects multiple LANs

- Supports global operations

Disadvantages:

- Expensive to install and maintain

- Lower speed compared to LAN

- More complex management

- Security concerns

Examples:

- The Internet (largest WAN)

- Bank networks connecting branches

- Multinational company networks

Key Differences Between LAN and WAN

| Feature | LAN | WAN |
| --- | --- | --- |
| Full Form | Local Area Network | Wide Area Network |
| Coverage Area | Small area | Large area |
| Speed | High | Lower |
| Ownership | Private | Private or public |
| Cost | Low | High |
| Security | More secure | Less secure |

### Conclusion

- LAN is suitable for small, localized networks requiring high speed and security.

- WAN is used for large-scale networks that connect multiple LANs across long distances.

6) Explain Operation of a Switch

ANSWER : Operation of a Network Switch

A network switch is a Layer 2 (Data Link Layer) device used in LANs to connect multiple devices and efficiently forward data to the correct destination.

How a Switch Works (Step-by-Step)

1. Frame Reception

- A switch receives a data frame from a connected device on one of its ports.

- The frame contains source and destination MAC addresses.

2. MAC Address Learning

- The switch checks the source MAC address of the incoming frame.

- It stores this MAC address along with the port number in its MAC address table (also called CAM table).

- This process is called learning.

3. Frame Forwarding / Filtering

- The switch checks the destination MAC address:

  - If the address is in the MAC table → the frame is forwarded only to the correct port.

  - If the address is not in the MAC table → the switch performs flooding (sends the frame to all ports except the incoming one).

  - If source and destination are on the same port → the frame is filtered (not forwarded).

4. Frame Transmission

- The frame is transmitted through the selected port to the destination device.

- This reduces unnecessary network traffic.

Switch Operation Model

1. Store-and-Forward

- The switch receives the entire frame.

- Checks for errors using CRC.

- Forwards only error-free frames.

2. Cut-Through

- The switch starts forwarding as soon as the destination MAC is read.

- Faster but may forward corrupted frames.

Key Features of a Switch

- Operates at OSI Layer 2

- Uses MAC addresses

- Reduces network collisions

- Improves bandwidth usage

- Supports full-duplex communication

Advantages of a Switch

- Faster than a hub

- More secure (sends data only to intended device)

- Efficient use of network bandwidth

### Conclusion

A switch intelligently forwards data frames based on MAC addresses, making LAN communication faster, more reliable, and more secure.

7) Describe the Purpose and Functions of Network Devices

ANSWER : Purpose and Functions of Network Devices

Network devices are hardware components used to connect computers and other devices, control data flow, improve performance, and ensure secure communication in a network.

1. Hub

Purpose:
To connect multiple devices in a network.

Functions:

- Broadcasts data to all connected devices

- Operates at OSI Layer 1 (Physical Layer)

- Does not filter or manage traffic

2. Switch

Purpose:
To efficiently connect devices within a LAN.

Functions:

- Forwards data based on MAC addresses

- Reduces network congestion

- Operates at OSI Layer 2 (Data Link Layer)

3. Router

Purpose:
To connect different networks together.

Functions:

- Routes data based on IP addresses

- Determines best path for data transmission

- Operates at OSI Layer 3 (Network Layer)

4. Modem

Purpose:
To connect a network to the Internet.

Functions:

- Modulates and demodulates signals

- Converts digital signals to analog and vice versa

5. Bridge

Purpose:
To divide a LAN into smaller segments.

Functions:

- Filters traffic using MAC addresses

- Reduces collisions

- Operates at OSI Layer 2

6. Repeater

Purpose:
To extend network distance.

Functions:

- Regenerates and amplifies signals

- Operates at OSI Layer 1

7. Gateway

Purpose:
To connect networks using different protocols.

Functions:

- Protocol conversion

- Acts as an entry/exit point for a network

8. Access Point (AP)

Purpose:
To provide wireless network access.

Functions:

- Connects wireless devices to a wired network

- Manages Wi-Fi communication

Summary Table

| Device | Main Purpose | OSI Layer |
| --- | --- | --- |
| Hub | Broadcast data | Layer 1 |
| Switch | Forward frames | Layer 2 |
| Router | Route packets | Layer 3 |
| Modem | Internet access | Layer 1 |
| Bridge | Segment network | Layer 2 |
| Repeater | Signal extension | Layer 1 |
| Gateway | Protocol conversion | Multiple |
| Access Point | Wireless access | Layer 2 |

### Conclusion

Each network device has a specific purpose and function that contributes to efficient, secure, and reliable network communication.

8) List Media, Cables, Ports, and Connectors Used to Connect Switches

ANSWER : Media, Cables, Ports, and Connectors Used to Connect Switches

Switches can be connected using different transmission media, cables, ports, and connectors, depending on distance, speed, and network type.

1. Transmission Media

a) Copper Media

- Uses electrical signals

- Suitable for short distances

Examples:

- Twisted Pair cable

- Coaxial cable

b) Fiber Optic Media

- Uses light signals

- Suitable for long distances and high speeds

Examples:

- Single-mode fiber

- Multi-mode fiber

2. Cables Used

a) Twisted Pair Cables

- UTP (Unshielded Twisted Pair)

- STP (Shielded Twisted Pair)

Common Categories:

- Cat5e

- Cat6

- Cat6a

Uses:

- Connecting switches to computers

- Switch-to-switch connections

b) Fiber Optic Cables

- Single-mode fiber (long distance)

- Multimode fiber (shorter distance)

Uses:

- Backbone links

- High-speed switch interconnection

c) Coaxial Cable (Rare)

- Older networks

- Limited modern use

3. Ports Used on Switches

- Ethernet (RJ-45) Ports – Most common

- SFP / SFP+ Ports – For fiber connections

- QSFP Ports – High-speed (40G/100G)

- Console Port – For switch configuration

4. Connectors Used

a) RJ-45 Connector

- Used with twisted pair cables

- Most common Ethernet connector

b) Fiber Connectors

- LC Connector

- SC Connector

- ST Connector

c) Transceivers

- SFP / SFP+ modules

- Convert electrical signals to optical signals

Summary Table

| Category | Examples |
| --- | --- |
| Media | Copper, Fiber Optic |
| Cables | UTP, STP, Fiber Optic |
| Ports | RJ-45, SFP, SFP+, QSFP |
| Connectors | RJ-45, LC, SC, ST |

### Conclusion

Switches are interconnected using appropriate media, cables, ports, and connectors based on required speed, distance, and network design.

9) Define Network Devices and Hosts

ANSWER : Network Devices: Hardware that manages or directs network traffic
(Switch, Router, Firewall, Access Point)

Network devices are hardware components that connect, manage, and control data transmission between computers and networks.

Definition:

Network devices are intermediary hardware used to transmit, route, filter, or manage data traffic in a network.

Examples:

- Hub

- Switch

- Router

- Bridge

- Repeater

- Modem

- Gateway

- Access Point

Purpose:

- Control data flow

- Improve performance

- Connect multiple networks

- Enhance security and reliability

- Hosts: End devices that send or receive data
(PCs, Laptops, Servers, Printers, Smartphones)

Hosts are end devices that send or receive data on a network.

Definition:

A host is any device connected to a network that has an IP address and participates in communication.

Examples:

- Computers (PCs, laptops)

- Servers

- Smartphones

- Printers

- IP cameras

Purpose:

- Generate and consume data

- Provide services (e.g., file server, web server)

Key Difference Between Network Devices and Hosts

| Feature | Network Devices | Hosts |
| --- | --- | --- |
| Role | Intermediary | End device |
| Main Function | Manage data traffic | Send/receive data |
| IP Address | Usually no (except routers) | Yes |
| Examples | Switch, Router | PC, Server |

### Conclusion

- Network devices manage and control network communication.

- Hosts are the source and destination of data in a network.

Top of Form

Bottom of Form